M5F-Built-in Battery-Quanzhi

Product Specification (Model: M5F-内置电池 - 全志)

Product Information

| Appearance Name | M5-Quanzhi 851 Explosion-proof Edition |
| --- | --- |
| Series Model | M5F-内置电池 - 全志 |
| Features | It has explosion proof certification. 3. Mass production can be IP68 4. H265 compression 5. Support airbag safety. The product needs to have a G sensor that can record drops and record collisions. 6. There is a report from the Ministry of Public Security 7. Optional WiFi and GPS |

Six Views

Function Diagram

Basic Information

| processor | Quan Zhi 851 |
| --- | --- |
| operating system | Linux |
| Screen size | 2.0 inch vertical screen |
| Default battery capacity | 4000mAh |
| Optional battery capacity upgrade | no |
| charging duration | 3 hours |
| Continuous recording duration | about 18 hours |
| standby time | about 48 hours |
| anti-shake | no |
| image format | MP4 |
| compression format | H.264/H.265 |
| Video size | Maximum 4K(3840*2160)30FPS |
| photo format | JPEG |
| horn | 1W |
| body memory | Scalable up to 512G |
| Rear camera (main) | 400W |
| Photo size | Maximum 58 million |

Lighting

| laser lamp | Support auxiliary locking imaging center or laser marking |
| --- | --- |
| white light Lamp | have |
| alarm lamp | Red and blue warning lights |
| infrared lamp | the 4 |

Interfaces

| charging interface | Type-C |
| --- | --- |
| G-sensor | support |
| vibration motor | support |
| external camera | support |
| charging method | Complete line charging |
| charger | 5V-2A |
| USB interface | USB 2.0 |

Data Functions

| WIFI | Point-to-point AP-wifi is optional |
| --- | --- |
| Bluetooth/433 | no |
| positioning | no |

Dimensions

| Length (mm) | 63 |
| --- | --- |
| Height (mm) | 89 |
| Thickness (mm) | 37 |
| Weight (g) Approx | 160 |

Basic Device Functions

| Video pre-recording | support |
| --- | --- |
| Video extension | support |
| Impact alarm | Support airbag security. The product needs to have a G sensor, which can record drops and record collisions. If it has a 4G function, it can be linked in the background. |
| night vision | Automatic or manual night vision switching, support for high-power red lights |
| fill light | Support the use of white LEDs for auxiliary lighting in dark situations. |
| USB flash drive function | Drive mode: After connecting to the computer, you can enter the USB flash drive mode (removable hard disk) through the upper computer on the PC side; Non-drive mode: After connecting to the computer, directly enter USB flash drive mode (removable hard disk); one of the two modes can be selected |
| menu language | Default Simplified Chinese/English (other languages can be customized) |
| Key Document Marker | You can mark this video file by pressing the corresponding button during the recording process; |
| continuous shooting function | The supporting device can take more than 10 photos continuously at one time |
| password protection function | support |
| Close beat mode | Support no indicator light display and no sound reminder mode. |
| Shooting in the middle | During the recording process, press the shoot button to take a photo individually |
| video encryption | not support |
| automatically overwritten | Support automatic override/non-override |
| in-vehicle functions | Support vehicle-mounted certificate collection, and realize automatic video recording and evidence collection when the car ignites and turns on, and automatic shutdown after parking and flameout. |
| safe mode | Safe mode is limited to basic operations such as recording, taking photos, and recording. A password is required to enter the settings interface and playback interface. |
| acquisition station | Support collection station and background management 1. Operation logs;2. Data collection and classification |
| data security management | Law enforcement recorders have different authority management. Primary authority can only view and playback law enforcement videos, photos, and recordings, and cannot delete or modify sensitive settings of the machine. Sensitive settings of the machine can only be made through password verification, and can only be copied, modified or deleted after connecting to a computer. |
| motion detection | Supports automatic recording and stopping of recording based on scene changes and still |
| One-click shortcut function | Support one-click quick start of recording, one-click start of recording function (no need to start the power button separately) |

Others

| protection grade | IP68 fell 2 meters |
| --- | --- |
| injection molding process | Two-color mold process (anti-crack) |
| low-temperature compatibility | Corresponding low-temperature batteries should be prepared for inspection and low-temperature areas. The low-temperature-30 ℃ battery life of a single battery should be more than 2 hours. Normal temperature batteries should be standard. Normal temperature batteries and low-temperature batteries have different densities and operating environments. |
| Storage relative humidity | 63%(30℃) |
| storage temperature | -40~60℃ |
| operating relative humidity | 40%~80% |
| operating temperature | -20~55℃ |

Packing Information

| Box gauge: length × width × height mm |  |
| --- | --- |
| Weight of the whole box: |  |
| Number of boxes: | -------------------- |
| Package: L × W × H mm |  |
| Net weight of packaging box: |  |
| Machine net weight: |  |
| Weight of outer packaging box: | 550g |

Standard Accessories

Dedicated Optional Accessories

Generated At: 2026-08-21 00:44:12 | Specification Version: M5F-Built-in Battery-Quanzhi